Yeast Protein Isolation

All reagents should be at 4C and samples handled on ice until 2min spin

Spin down 1-1.5x107 log-phase cells in a 2ml tube, wash cells with 1 ml water

You can freeze the cell pellet in N2 and process later

This can be scaled up 5x – increase the volume of all the reagents by 5x except the acetone, do two washes with 1ml of acetone*

*for low cell number process samples straight after MEP Rapid Aging Purification, include 1x cOmplete Protease Inhibitors in buffers during MEP purification

Re-suspend cell pellet in 100l water

Add 15l 2N NaOH w/ 80mM DTT (freshly added), vortex, ice for 10 minutes

Add 15l 50% TCA, vortex, ice for 10 minutes

Samples can be held indefinitely at 4 at this point

Spin 2min 14,000rpm at RT

Pipette off supernatant into TCA waste, add 1ml ice-cold acetone

Invert to mix then spin 2min 14,000rpm at RT

Pipette off acetone and dry briefly

For normal samples

Re-suspend pellet in 40l sample buffer, vortex and heat at 65 for 5 min

Vortex again until pellet is gone (pipetting can help too)

Heat 5 min at 65 again

For low cell number

Resuspend in 20l  of Laemlli buffer containing freshly added 100mM DTT

Heat 5 mins at 95°C

Spin the sample for 30s at top speed and transfer supernatant to new tube, store -20

Quantification and loading:

Measure concentration by OD280. Use buffer including DTT as a reference since DTT absorbs at this wavelength

Add 10x Orange G to each sample to 1x final concentration before loading

For samples in Sample buffer:

Heat 10min at 65, spin briefly and load on gel

(Don’t place samples on ice as they will solidify!)

For samples in Laemlli buffer:

Heat 5min at 95, spin briefly and load on gel

Sample buffer:					10ml :

70mM Tris pH6.8	700µl of 1M

8M urea		4.8g

5% SDS		2.5ml 20% (the good stuff from Biorad)

1mM EDTA		20µl of 0.5M

water to 9ml, warm gently to dissolve

100mM DTT		Add from 1M stock just before use

Laemlli buffer:           60mM Tris pH 6.8

10% Glycerol

2% SDS (from the high quality 20% stock)

Make this to 90% final volume, then just before use add

10% 1M DTT (giving final concentration of 100mM DTT)

10x Orange G:		0.1% Orange G in water

7.4% β-mercaptoethanol can be used, but the process needs to be performed in the fume hood

don’t heat samples above 70 in this buffer as the proteins will aggregate

Diluting and measuring by Bradford assay is probably more accurate